Unravelling the Unusual Chemistry of the Hydrogen Peroxide-Driven Hypergolic Ignition of a Cyanoborohydride Ionic Liquid as Next-Generation Green Space Propellant

Souvick Biswas1,$, Kazuumi Fujioka1,$, Dababrata Paul1, Mason Mcanally1, Grace L. Rizzo1, Steven D. Chambreau2, Stefan Schneider2, Rui Sun1*, Ralf I. Kaiser1*

1 Department of Chemistry, University of Hawai’i at Manoa, Honolulu, Hawaii 96822, United States

2 Air Force Research Laboratory, Edwards Air Force Base, California 93524, United States

* Corresponding authors. E-mail: ruisun@hawaii.edu, ralfk@hawaii.edu

$Authors contributed equally

Abstract

Hypergolic ionic liquids (HIL) have emerged as promising self-igniting green space-propellants in combination with an oxidizer replacing toxic hydrazine-family rocket fuels. Despite numerous new HILs being reported in literature, there is no systematic study addressing the key reaction mechanism of such hypergolic ignition. Here, the maiden comprehensive molecular level understanding of this ignition reaction is revealed exploring 1-ethyl-3-methylimidazolium cyanoborohydride - hydrogen peroxide ([EMIM][CBH]-H2O2) green bipropellant pair by a novel chirped-pulse droplet-merging technique in a controlled environment. Mechanistically, the anion [CBH]- triggers the hypergolic ignition through facile exoergic oxidation of the boron-center yielding boron dioxide (BO2) in a barrierless termolecular reaction with two molecules of H2O2 followed by an enhanced reactivity of the cation [EMIM]+ as evidenced from the excess yield of carbon dioxide (CO2) and evaluated decay rate constants of [EMIM]+ in situ during droplet-merging reaction.

TOC Graphic

Since the elucidation of the mathematical concept of rocket propulsion by Robert H. Goddard in the early 20th century, scientists pursue the goal to operate faster rockets and reach distant regions of space beyond the boundary of Earth.1 After decades of research, hypergolic or self-igniting bipropellants have become a popular choice for distant space missions circumventing the limitations and complexities of cryogenic propulsion systems.2 A traditional hypergolic propellant consists of a fuel, which ignites spontaneously (without any ignition source) upon contact with an oxidizer followed by generating superior thrust and excellent combustion performance. Unfortunately, common hypergolic fuels, for example, hydrazine (N2H4) and its derivatives such as methylhydrazine (H2NNH(CH3)), unsymmetric dimethyl hydrazine (UDMH, (CH3)2N2H2)) used in combination with an oxidizer like white fuming nitric acid (WFNA, HNO3) or dinitrogen tetroxide (N2O4) are highly toxic, volatile and carcinogenic.3, 4

In a major scientific breakthrough by Schneider et al. in 2008, specific ‘ionic liquids’ – salts that are liquid at room temperature  - were demonstrated to be hypergolic by testing the synthesized 1-allyl-3-methylimidazolium dicyanamide with WFNA as oxidizer.5 This introduction to hypergolic ionic liquids (HIL) has revolutionized the development of green bipropellants as potent replacements of the harmful hydrazine family fuels. Numerous HILs have been synthesized composed of distinct classes of anions and cations in order to enhance reactivity and performance.6-28 In general, the anions are shown to initiate and promote the hypergolic ignition.29 Thus, key electron and fuel-rich anions (e.g. dicyanamide [N(CN)2]-, nitrocyanamide [N(NO2)(CN)]-, borohydrate [BH4]-, cyanoborate [BHn(CN)4-n]- (n = 1-3)) which can be easily oxidized have been synthesized with the high energy density boron-centered anions exhibiting superior ignition performance.30

Here, we explore the hypergolicity of such an energetic ionic liquid: 1-ethyl-3-methylimidazolium cyanoborohydride ([EMIM][CBH]) with concentrated hydrogen peroxide (H2O2, ≥ 80% w/v) (Figure 1). The constituent ions of the ionic liquid (IL) are purposefully selected. First, cyanoborohydride ([BH3(CN)]-)-based ILs have exceptional thermal stabilities, moisture-insensitivity and high energy-content when compared to energy-rich borohydrates.14 Second, imidazolium-based ILs possess greater stability than their triazolium or tetrazolium analogues and are suitable for flexible design and synthesis. In particular, [EMIM]+ in dicyanamide (DCA) series of ILs has been shown to be most reactive among the imadzolium cations with saturated alkyl substitutions.31 Overall, [EMIM][CBH] possesses a high thermal stability, has an extremely low vapor pressure (as compared to hydrazine family fuels), a preferable density (0.98 g cm-3) and a low viscosity (19 mPa-s).14, 30 Low viscosities are essential for the contact and mixing of reactants, which makes it a viable fuel for convenient engine operations. Environmentally unfriendly oxidizers such as white/red fuming nitric acid (HNO3) or dinitrogen tetroxide (N2O4) are replaced with a ‘clean’ and ecofriendly alternative - hydrogen peroxide (H2O2)32-35, making the reactant pair ([EMIM][CBH]-H2O2) substantially much ‘greener’ compared to common hypergols.

Here, we report the first comprehensive combined experimental and computational study to unravel the hypergolic ignition chemistry exploiting the [EMIM][CBH] - H2O2 bipropellant pair. It is worth mentioning that the liquid fuel (ionic liquid) and the oxidizer initially interact at the droplet level inside the combustion chamber of rocket36, 37, unlike the usual drop tests performed which have significant surface contribution and excess of oxidizer content, thus do not reflect true ignition delay values and vary largely within different experimental groups. Therefore, the present experiments are performed systematically under controlled conditions in a closed process chamber filled with argon exploiting a novel droplet merging method, where quantifiable individually levitated droplets of the HIL and the oxidizer are merged via frequency chirped amplitude modulation of the carrier wave in an ultrasonic levitator apparatus38, 39 (Supporting Information S1). Multiple synchronized spectroscopic tools have been utilized in conjunction for the sensitive detection of the gaseous products (Fourier-transform infrared (FTIR) spectroscopy) and reaction intermediates (time-resolved UV-Vis emission and Raman spectroscopy)40-44 revealing boron dioxide (BO2) to be the key emitting intermediate driving the hypergolic ignition as evident from a bright green flame. To unravel the specific roles of the anion and cation during the oxidation, the reaction has been slowed down by diluting the oxidizer (H2O2) – thus enabling the identification of the initial reaction intermediates formed in situ (Raman spectroscopy). Extensive state-of-the-art electronic structure theory calculations (Supporting Information S2) complementing the experiments identify the origin of hypergolicity at the molecular level along with the reaction mechanism for the formation of stable products and intermediates.

Hypergolic Ignition via Chirped Pulse-Triggered Droplet Merging

The merging process of the droplets of [EMIM][CBH] and 92% H2O2 loaded in the lower and upper pressure node, respectively, in the standing wave generated in the acoustic levitator, followed by the ignition event was monitored by the high-speed, temporally resolved, synchronized optical and infrared cameras (Figure 1, Movies S1, S2). Series of snapshots depict distinct stages comprising of (i) levitating reactant droplets (pre-merging), (ii) merging motion upon application of the frequency-chirp pulse, and (iii) merging instance accompanied by temperature rise (exoergic) with subsequent (iv) initiation and (v)-(vi) progress of the ignition (combustion) events revealing bright greenish flames with yellowish tints (Figure 1).

Figure 1. (a) The hypergolic bipropellant 1-ethyl-3-methylimidazolium cyanoborohydride ([EMIM][CBH]) and concentrated H2O2 (≥ 80% w/v). Synchronized, temporally resolved (b) optical and (c) infrared camera images of the droplet merging event followed by ignition between [EMIM][CBH] and 92% w/v H2O2. The evolution of the ignition is spectroscopically probed by recording (d) time-resolved emission (UV-Vis) spectra. (e) Deconvolution of the most intense flame emission spectrum with the corresponding optical image depicted in the inset.

Dense smoke formation is associated with the initiation of the ignition (iv), and the temperature exceeds 2,500 K upon expansion of the flame (Supporting Information S3). An ignition delay (ID) of 390 ± 10 ms is determined from the time difference between the merging and the initiation of ignition. Further, testing the IL ([EMIM][CBH]) with diluted H2O2 (10 to 80%) revealed the threshold oxidizer concentration for the hypergolicity of this bipropellant pair to be 80% (Figure S6, Movies S3, S4). Consequently, the noted ignition delay (ID) is lengthened to 520 ± 10 ms as compared to that involving 92% H2O2 suggesting a direct concentration dependence of the oxidizer. The molar concentration of H2O2 is required to be about four times with respect to the IL for the occurrence of these hypergolic reactions as evaluated from the quantification of the reactant droplets (Table S1).

Flame Emission Spectra

The ignition chemistry was probed spectroscopically exploiting ultraviolet – visible (UV-Vis) emission spectroscopy in the 200 to 1,100 nm spectral range; the time-resolved emission spectra are displayed in Figure 1(d). The temporal evolution of emission spectral intensities indicates the commencement of the ignition, the spread of the flame to its peak intensity, i.e. the combustion of the ionic liquid droplet, followed by no emission implying the completion of the oxidation process. This time sequence as perceived spectroscopically also mirrors the recordings of the optical camera (Figure 1(b)). The deconvolution of the emission traces (Figure 1(e), Table S2 and Figure S7) reveals dominant emission features of boron dioxide (BO2) radical (548.4 nm; A 2Πu – X 2Πg)45, 46. The strong presence of BO2 is attributed to the facile oxidation of sp3 hybridized B-center of the [CBH] anion. However, another probable emitting intermediate, boron monoxide (BO)47, could not be detected in the emission spectra owing to the intense emissions from boron dioxide. Traces of alkali metals present in the IL are also detectable via their characteristic atomic emission lines of sodium (Na) [589 nm, 2p63p 2P → 2p63s 2S; 819 nm, 2p63d 2D → 2p63p 2P ] and potassium (K) [769 nm, 3p64p 2P → 3p64s 2S].48 Temporal profile of the reactive intermediate BO2 (Figure S8) reveals a more or less asymmetric bell-shaped curve with the peak around 10 ms and the strong ignition lasted about 20 ms. On the contrary, the ignition with 80% H2O2 was found to be short-lived (Figure S6), suggesting that the oxidation of the B-center of the [CBH]- is facilitated by higher concentration of the oxidizer.

Gas Phase Products

Irrespective of the ignition, the [EMIM][CBH]-H2O2 merging process was accompanied by the generation of gaseous products and those were detected via FTIR spectroscopy (Figure 2, Figure S9, Table S3). Since the oxidizer contains water as the medium, the broad features (stretching: 3950-3490 cm-1; bending:1910-1320 cm-1) of water vapor are obvious in the FTIR spectra. The primary combustion product, carbon dioxide (CO2), is detected from its strong asymmetric stretching (2350 cm-1) and bending vibration (667 cm-1) along with carbon monoxide (CO) by its stretch at 2143 cm-1. Characteristic features for hydrogen cyanide (HCN, 3311 and 712 cm-1), isocyanic acid (HNCO, 2269 cm-1), methane (CH4, 3150 – 2870 cm-1) and ammonia (NH3, 950 cm-1) are identified. HCN and HNCO plausibly originate upon decomposition and oxidation of the –C≡N functional group existing in [CBH]-. The strong presence of CO2 in the spectra suggests an involvement of the carbon-rich cation. In the analogous non-hypergolic reaction of the same IL with WFNA, no CO2 was formed, thus assuring a significant reactivity of cation towards H2O2.39 Note that the quantification of the major gaseous products (Figure 2(b)) from the calibration curves (Figures S10–S13) shows that their amounts (CO2, CO, HCN and CH4) increase with the rise in oxidizer concentration as a general trend, indicating greater decomposition of the IL. The yield of the predominant product, CO2 reaches the maximum conversion efficiency for the hypergolic reaction with 92% H2O2 of 59% from [EMIM]+ assuming the latter to be its primary source because of the six constituent carbon atoms (Table S4). A slight decrease in the quantity of HCN is observed while increasing the H2O2 concentration from 80 to 92% which can be linked to the subsequent oxidation of HCN49.

Figure 2. (a) FTIR spectra of the gas phase products formed during the reaction of ionic liquid [EMIM][CBH] and selected concentrations of hydrogen peroxide (H2O2) with colored segments for the detected species. Water features are subtracted from the 1600-1200 cm-1 segment to reveal bands of methane (CH4) and hydrogen cyanide (HCN). (b) Quantification of the major gaseous products during the reaction of ionic liquid [EMIM][CBH] and distinct concentrations of H2O2. The shaded area represents hypergolic reactions. The lower panel shows the percentage conversion of carbon dioxide (CO2) from the [EMIM]+ ion.

Reaction Kinetics from Raman Spectroscopy

The stepwise chemical changes for determining the underlying reaction mechanism have been investigated in situ Raman spectroscopically by slowing down the reaction using diluted H2O2 (10 - 50%). These merging processes are accompanied by gradual temperature increase with time and gas release via bubble formation in the droplet (Figure S15). The Raman spectra (Figures S14-S16) of the merged droplet reveal several new features (Table S6) of oxygenated intermediates compared to the reactants upon thorough spectral deconvolution, such as cyanato (N≡C−O) isocyanato (N=C=O), boron hydroxy (B-OH), orthoborate (BO3) and B(O-O)B linkage.

The reactivities of the IL and oxidizer have been quantified through the derivation of the decay rate constants as extracted by individual first order fitting of the time-dependent characteristic Raman peak areas (normalized) during the reaction occurring in the merged droplet (Figures S17, S23-S26). Similarly, the temporal evolution and the rate constants of formation/decay of the intermediates are also determined (Figures S18, S23-S26). From the summarized rate constants as a function of H2O2 concentration (Figure 3), it is apparent that H2O2 decays faster than both the anion and cation at a given concentration, and the decay is significantly accelerated upon increasing its concentration.

Figure 3. Rate constants of the (a) reactants and (b) products/intermediates with respect to the percentage concentration of H2O2 evaluated by the kinetic fitting of the decay or formation of the characteristic peaks over time as extracted from the consecutive Raman spectra (Figure S15-S26).

On the other hand, [CBH]- is found to be consistently more reactive (0.104 ± 0.019 s-1) in comparison to [EMIM]+ (0.037 ± 0.014 s-1) and measured to be about three times faster at the oxidizer concentration of 50%. The cumulative decay rate constants for the IL (sum of respective rate constants of anion and cation) follow a similar trend with the decay rates of H2O2, but the latter increases rapidly when the oxidizer concentration exceeds 40%. A relevant comparison of similar reactions of this IL with 30% H2O2 and HNO3, individually, reveals that the decay rate constant of [EMIM]+ relative to [CBH]- is much faster in the former (0.018 ± 0.01 s-1 versus 0.04 ± 0.02 s-1) than the latter (0.007 ± 0.003 s-1 versus 0.1 ± 0.01 s-1)38. This observed phenomenon ensures the exclusive reactivity of the cation to the present oxidizer (H2O2) quantitatively.

The appearance of the spectral features corresponding to the intermediates at the merging instance suggests that those are formed immediately. From the temporal profiles of these intermediates (Figures S18, S23-S26), it is evident that the boron-containing species continue forming with the progress of reaction, whereas intermediates bearing cyanato (N≡C−O) and isocyanato (N=C=O) groups undergo decay for the same. The formation/decay rates for all functional groups have been found to be strongly dependent upon H2O2 concentration (Figure 3(b)) and at higher oxidizer concentration (≥ 40%), the magnitude of the formation rate constants for boron containing intermediates exceed the decay constants of cyanato (N≡C−O) and isocyanato (N=C=O) group-bearing intermediates.

Electronic Structure Calculation

To explore the reaction mechanism of the hypergolic [EMIM][CBH]-H2O2 system, the experimental results were merged with extensive electronic structure calculations performed at the B2PLYP/cc-pVTZ//B3LYP/6-31G* level of theory for determining the zero-point energies of reactants, intermediates, transition states and products (Supporting Information S2). Note that, the computational treatment considers the whole ionic liquid, where the reactive ion (anion/cation) is only shown or discussed in the figures for simplicity, while the non-reacting one (cation/anion) remains spectator in the potential energy surfaces. The PES for the simplest bimolecular reactions between [CBH]- or [EMIM]+ with H2O2 (Figures 4, S27) reveal that although the corresponding entrance channels consist of energetically favored van-der-Waals complexes (i1, i4), the subsequent steps of hydroxyl (OH) reaction with the respective reactants (i1→i2, i4→p3) possess a significant barrier. The magnitude of the barrier with respect to the separated reactants is significantly higher for the cation (210 kJ mol-1) compared to the anion (35 kJ mol-1); this finding is consistent with the experimentally observed greater reactivity of [CBH]- than [EMIM]+ (Figure 3). Upon B-C bond cleavage of HOBH2CN- (i2→c1), a H-bonded complex of hydroxyborane (H2BOH) and CN– is formed. Consequently, HCN can be formed easily by either proton transfer from hydroxyborane (H2BOH) (c1→p2) or through stepwise complexation (c1→i3) and proton transfer from water (i3→p1) to CN–, while the former path (c1→p2) is exoergic by 203 kJ mol-1 and preferred to the other one.

Figure 4. Potential energy surfaces (PES) for the bimolecular reactions of (a) [CBH]- and (b) [EMIM]+ with H2O2, respectively.

However, PES (Figure 5a) depicting the oxidation of [CBH]- by two molecules of H2O2 forming boron dioxide (BO2) is found to be entirely submerged compared to the separated reactants. The initial van-der-Waals complex (i5, -131 kJ mol-1) undergoes a concerted OH-reaction substituting the H-atom at the B-center generating a H-bonded intermediate (i6) traversing the global maximum in the PES (-33 kJ mol-1). Intermediate i6 eventually releases a hydrated hydroxyl anion (H2O…OH-) via subsequent hydride elimination from the B-center to form a bound complex (i7, HB(OH)CN…H2O).

Figure 5. Potential energy surfaces (PES) for the termolecular reactions of (a) [CBH]- and (b) [EMIM]+ with two molecules of H2O2, respectively.

Through stepwise B-C bond cleavage (i7→i8), recombination (i8→i9) and isomerization (i9→i10), a new B-N bond is formed (i10, (HO)2BNHCH), which is ruptured in the next step to produce metaboric acid (HOBO) and methanimine (H2C=NH), (i11). Species in i11 are stabilized via the formation of a H-bonded complex, i12. Upon an intermolecular H-atom transfer in i12, i13 (CH3NH…BO2) is formed, which finally generates doublet radicals: boron dioxide (BO2) and methylamino (CH3NH) (p4). Alternatively, i12 can also produce BO2 and aminomethyl radical (CH2NH2) via the decomposition of the newly formed O-C linkage in OBOCH2NH2 (i14→p5). Ammonia (NH3), an experimentally detected gas phase reaction product, can be liberated from i12 as well via B-C bond formation along i12→i15 followed by a concerted C-N bond cleavage and H-atom transfer forming a van-der-Waals complex of NH3 and a cyclic heteroatomic species (HO-BCO), (i16) which is then cleaved to the final products. This PES noticeably outlines the facile oxidation mechanism of the B-center, ultimately leading to the formation of BO2, which is a predominant emitting intermediate as revealed experimentally, drives the ignition reaction. Evidently, all of the product formation (r3→p4, p5, p6) channels are exoergic by more than 200 kJ mol-1 and the released energy can activate subsequent decomposition/oxidation steps yielding smaller end-products.

Similarly, exploring the oxidation of the C-center of [CBH]-, several pathways exist which form major gas phase products, such as CO2, CO, HNCO and functional groups of the intermediates as detected by Raman spectroscopy, e.g. -O-C≡N and -N=C=O are determined (Figure S28). These reaction channels are energetically favored and the energy maximum for the PES is only 4 kJ mol-1. However, the excess CO2 as quantified from the infrared spectroscopy explicitly suggests that oxidation of the cation must have a major contribution. Hence, a termolecular reaction of [EMIM]+ with two molecules of H2O2 has also been explored (Figure 5b). The entrance channel leads to the formation of a bound van-der-Waals complex (i17, -84 kJ mol-1), followed by a concerted peroxo (-O-O-) linkage cleavage of H2O2 and double OH-insertion in the adjacent carbon atoms of the imidazolium ring along i17→i18. This step is the energy maximum (172 kJ mol-1) of the PES and lowered by 38 kJ mol-1 than that of the [EMIM]+- H2O2 bimolecular reaction. Ring opening (i18→i19) takes place via C-N bond cleavage in the methyl substituted side, followed by oxidation to a carboxylic acid terminal group (i20). Decarboxylation from i20 yields CO2 by accessing an energy barrier of 277 kJ mol-1 (i20→p7). CO elimination from i19 can occur via an energy barrier of 286 kJ mol-1 (i19→p8). The oxygenated two-carbon chain gets detached through a subsequent C-N cleavage along i19→i21 path forming glyoxal (CHO-CHO) and (E)-N-((methylamino)methylene)ethanaminium cation, where the latter undergoes a C-C cleavage on the ethyl chain to produce CH4 (p9). Among all the exoergic products (CH4, CO and CO2) in the PES, CO2 formation path emerges to be most favored and depicts the global minimum (-632 kJ mol-1) as well. Finally, the extensive oxidation of the residual ethanaminium-containing cation during the exothermic reaction is likely to produce CO and CO2 as stable end-products.

It can be easily noted that the reaction of [CBH]--2H2O2 system involves a submerged barrier and is energetically favored unlike the analogous reaction of [EMIM]+. This spontaneous reaction sequence consisting of multiple exoergic product formation channels for the oxidation of anion in turn results into the evolution of initial exoergic mixing enthalpy due to droplet merging (Table S9) ranging up to 11 kJ mol-1. On the other hand, the high global maximum (172 kJ mol-1) in the PES of [EMIM]+-2H2O2 system is deemed to be unsurmountable. Alternatively, the high energy transition state (i17→i18) can be attained easily from the exoergic product formation steps ranging 139-615 kJ mol-1 (Figures 5(a), S28) in the termolecular reactions of [CBH]--2H2O2 system. The abovementioned finding from energetic viewpoint is also manifested in the sluggish rate constants (Figure 3) for the decay of [EMIM]+ (0.037 ± 0.014 s-1) than that of the anion (0.104 ± 0.019 s-1) while reacting with H2O2 (rates mentioned in case of 50 %) as determined from the Raman spectroscopy data. At the molecular level, it can be inferred that the IL-H2O2 reaction is initialized by [CBH]- via the facile stepwise oxidation of B-center followed by the participation of the cation through the simultaneous oxidation of adjacent unsaturated C-atoms of the imidazolium ring; overall, the elementary reactions of the anion and cation, individually are not simultaneous, but occur in consecutive steps. Considering both the termolecular reactions, four molecules of the oxidizer are consumed per IL molecule, which is consistent with the measured molar ratio of the IL to oxidizer for the occurrence of spontaneous hypergolic ignition. In a much-complicated course of reaction (Figure S29), extensive oxidation of [CBH]- leads to the highly exoergic formation of boric acid (H3BO3, i40) and coupling of two B-centers through a peroxo linkage is also found to be feasible (HOBHOOBHOH, p15). Vibrational features for both of these have been identified from the Raman spectra.

This combined experimental and computational study of the [EMIM][CBH]-H2O2 bipropellant system by a novel chirped-pulse droplet-merging technique in a controlled environment reveals the hypergolic ignition is originated via the facile exoergic oxidation of the boron-center in the anion [CBH]- upon a barrierless termolecular reaction with two molecules of H2O2 followed by oxidation of adjacent unsaturated carbon atoms of the imidazolium ring and a subsequent C-N bond cleavage in the cation [EMIM]+. The spontaneous oxidation of boron leading to the formation of BO2, the key emitting reactive intermediate was unambiguously detected by temporally-resolved emission spectroscopy. The yield of excess CO2 quantified by FTIR spectroscopy and moderate decay rate constants of [EMIM]+ extracted from Raman spectroscopy, even during the reaction with dilute H2O2 provides direct experimental evidence for the enhanced reactivity of the cation. However, the oxidizer (H2O2) concentration-dependent decay constants conclude that [CBH]- reacts faster than the cation, which is correlated to the high initial energy barrier for the latter contrary to the overall submerged potential energy surface for the former in case of the respective termolecular reactions. Pivotal initial oxidation steps of the IL were probed mechanistically by identifying the Raman vibrational frequencies for functional groups of intermediates such as cyanato (N≡C−O), isocyanato (N=C=O), B-OH and BO3 in the merged droplet in situ. Overall, this pioneering work represents the first ever systematic and novel experimental investigation to untangle the complex chemistry emanating hypergolicity of ILs. Further studies are aimed at exploring the structure–reactivity effects originating from distinct organic side groups in the cation for energetic cyanoborohydride ILs and finally extending to other energy-rich anion-containing ILs; thus, providing a comprehensive framework of reactivities dictating the hypergolicity of ILs at the molecular level, which is necessitated for the advancement of energy-efficient green hypergolic space propellants.

Supporting Information

The Supporting Information is available free of charge at https://pubs.acs.org.

Experimental and computational methods, temporally resolved optical images with simultaneous flame emission spectra for the hypergolic ignition reaction between [EMIM][CBH] and 80% H2O2, flame temperature determination, quantification of molar ratios, peak assignments for the deconvoluted flame emission spectra, temporal profiles of the key emitting intermediates, FTIR spectra of the gas phase products and assignments of the bands, calibration curves of the major gaseous products, percentage yield of carbon dioxide (CO2) from the [EMIM]+, deconvolution and assignment of the Raman spectra for the levitating H2O2 droplets, and merged droplet of IL-H2O2, temporal profiles, kinetic fitting and evaluated rate constants of the reactants and products during merging reactions probed by Raman spectroscopy, transition states for the reactions of [CBH]- and [EMIM]+ with H2O2, potential energy profile for the [CBH]- + 2H2O2 reaction forming CO2, CO, HNCO and OCN-, potential energy profile for extensive oxidation of [CBH]- by H2O2 forming H3BO3 and peroxo linkage (-O-O-) in HOBHOOBHOH, enthalpy changes during merging, coordinates of the reactants, intermediates, transition states and products, optical and infrared movies of droplet merging and ignition events.

Other Supporting Materials

Supporting Movies S1 to S6 for the droplet merging events of the IL with 92, 80, 50 and 20% H2O2.

Conflict of interest

The authors declare no conflict of interest.

Author Contributions

R.I.K. designed the experiments; S.B., M.M. and G.L.R. carried out the experiments; S.B. and D.P. analyzed the data; S.D.C. and S.S. carried out the synthesis; K.F. and R.S. performed the theoretical calculations; S.B. wrote the original draft, S.B., R.S. and R.I.K. revised the draft and all of the authors discussed the data. S.B. and K.F. contributed equally.

Acknowledgements

This work was supported by the Air Force Office of Scientific Research (AFOSR) (FA9550-21- 1-0377). The authors appreciate the Information Technology Service (ITS) from the University of Hawaii at Manoa for the computational resources.

References

(1) Lehman, M. Robert H. Goddard; Da Capo Press, 1988.

(2) Sutton, G. P. History of Liquid Propellant Rocket Engines; American Institute of Aeronautics and Astronautics, 2006.

(3) Nguyen, H. N.; Chenoweth, J. A.; Bebarta, V. S.; Albertson, T. E.; Nowadly, C. D. The Toxicity, Pathophysiology, and Treatment of Acute Hydrazine Propellant Exposure: A Systematic Review. Mil. Med. 2021, 186, e319-e326..

(4) Special Project: Fire, Explosion, Compatibility, and Safety Hazards of Nitrogen Tetroxide (AIAA SP-086-2001). In Special Project: Fire, Explosion, Compatibility, and Safety Hazards of Nitrogen Tetroxide (AIAA SP-086-2001).

(5) Schneider, S.; Hawkins, T.; Rosander, M.; Vaghjiani, G.; Chambreau, S.; Drake, G. Ionic Liquids as Hypergolic Fuels. Energy Fuels 2008, 22, 2871-2872.

(6) Gao, H.; Shreeve, J. n. M. Ionic Liquid Solubilized Boranes as Hypergolic Fluids. J. Mater. Chem. 2012, 22, 11022-11024.

(7) He, L.; Tao, G.-H.; Parrish, D. A.; Shreeve, J. n. M. Nitrocyanamide-Based Ionic Liquids and Their Potential Applications as Hypergolic Fuels. Chem. Eur. J. 2010, 16, 5736-5743.

(8) Lauck, F.; Balkenhohl, J.; Negri, M.; Freudenmann, D.; Schlechtriem, S. Green Bipropellant Development – A Study on The Hypergolicity of Imidazole Thiocyanate Ionic Liquids with Hydrogen Peroxide in an Automated Drop Test Setup. Combust. Flame 2021, 226, 87-97.

(9) Li, S.; Gao, H.; Shreeve, J. M. Borohydride Ionic Liquids and Borane/Ionic-Liquid Solutions as Hypergolic Fuels with Superior Low Ignition-Delay Times. Angew. Chem. Int. Ed. 2014, 53, 2969-2972.

(10) Schneider, S.; Hawkins, T.; Ahmed, Y.; Rosander, M.; Hudgens, L.; Mills, J. Green Bipropellants: Hydrogen-Rich Ionic Liquids that Are Hypergolic with Hydrogen Peroxide. Angew. Chem. Int. Ed. 2011, 50, 5886-5888.

(11) Wang, K.; Zhang, Y.; Chand, D.; Parrish, D. A.; Shreeve, J. n. M. Boronium-Cation-Based Ionic Liquids as Hypergolic Fluids. Chem. Eur. J. 2012, 18, 16931-16937.

(12) Zhang, Q.; Shreeve, J. M. Energetic Ionic Liquids as Explosives and Propellant Fuels: A New Journey of Ionic Liquid Chemistry. Chem. Rev. 2014, 114, 10527-10574.

(13) Zhang, Q.; Shreeve, J. n. M. Ionic Liquid Propellants: Future Fuels for Space Propulsion. Chem. Eur. J. 2013, 19, 15446-15451.

(14) Zhang, Q.; Yin, P.; Zhang, J.; Shreeve, J. M. Cyanoborohydride-Based Ionic Liquids as Green Aerospace Bipropellant Fuels. Chem. Eur. J. 2014, 20, 6909-6914.

(15) Zhang, Y.; Gao, H.; Joo, Y. H.; Shreeve, J. M. Ionic Liquids as Hypergolic Fuels. Angew. Chem. Int. Ed. 2011, 50, 9554-9662.

(16) Zhang, Y.; Shreeve, J. n. M. Dicyanoborate-Based Ionic Liquids as Hypergolic Fluids. Angew. Chem. Int. Ed. 2011, 50, 935-937.

(17) Mota, F. A. S.; Fei, L.; Liu, M.; Jiang, J.; Tang, C. Novel Hypergolic Green Fuels with Hydrogen Peroxide for Propulsion Systems. J. Propul. Power 2024, 40, 207-219.

(18) Mota, F. A. S.; Dias, G. S.; Fei, L.; Tang, C. Ignition Envelope and Bubbly Spray Combustion of a Cost-Effective Self-Igniting Fuel with Rocket-Grade Hydrogen Peroxide. Combust. Flame 2024, 270, 113744.

(19) Wang, B.; Wang, Z.; Jin, Y.; Wang, K. Designing Difunctional Promoters for Hypergolic Ignitions of Green Bipropellants Combining Ionic Liquids with H2O2. Ind. Eng. Chem. Res. 2022, 61, 17433-17439.

(20) Li, X.; Huo, H.; Li, H.; Nie, F.; Yin, H.; Chen, F.-X. Cyanotetrazolylborohydride (CTB) Anion-Based Ionic Liquids with Low Viscosity and High Energy Capacity as Ultrafast-Igniting Hypergolic Fuels. Chem. Comm. 2017, 53, 8300-8303.

(21) Pialat, A.; Kitos, A. A.; Witkowski, T. G.; Cook, C.; Wang, S.; Hu, A.; Murugesu, M. Achieving Short Ignition Delay and High Specific Impulse with Cyanoborohydride-Based Hypergolic Ionic Liquids. New. J. Chem. 2022, 46, 21212-21220.

(22) Li, J.; Weng, X.; Tang, C.; Zhang, Q.; Fan, W.; Huang, Z. The Ignition Process Measurements and Performance Evaluations for Hypergolic Ionic Liquid Fuels: [EMIm][DCA] and [BMIm][DCA]. Fuel 2018, 215, 612-618.

(23) Isaac Sam, I.; Gayathri, S.; Santhosh, G.; Cyriac, J.; Reshmi, S. Exploring the Possibilities of Energetic Ionic Liquids as Non-Toxic Hypergolic Bipropellants in Liquid Rocket Engines. J. Mol. Liq. 2022, 350, 118217.

(24) Mota, F. A. S.; Fei, L.; Tang, C.; Huang, Z.; Costa, F. S. Hypergolic Ignition Behaviors of Green Propellants with Hydrogen Peroxide: The TMEDA/DMEA System. Fuel 2023, 336, 127086.

(25) Mota, F. A. S.; Liu, M.; Mohsen, A. A. A.; Yao, X.; Mai, Z.; Tang, C. Development of Polyamine/Alkanolamine-Based Hypergolics with Hydrogen Peroxide: A New Route to N-methylimidazole with MDEA as a Promising Green Fuel. Fuel 2024, 357, 129798.

(26) Dias, G. S.; da Silva Mota, F. A.; Fei, L.; Tang, C.; de Souza Costa, F. Gelled Hydrogen Peroxide: Hypergolic Reaction with Low Toxicity Fuel by Drop and Impinging Jets Tests. Acta Astronaut. 2024, 222, 471-480.

(27) Mota, F. A. S.; Dias, G. S.; Machado, D. A.; Andrade, J. C.; Costa, F. S.; Fei, L.; Tang, C. Characterization of the Hypergolic Ignition of Green Fuels with Hydrogen Peroxide by Drop Test and Jet Impingement. Combust. Flame 2024, 267, 113567.

(28) Wu, Y.; Kong, X.; Ao, Y.; Mota, F. A. S.; Wang, J.; Zhang, Y.; Tang, C.; Huang, Z. Understanding the Synergistic Effect in Green Propellants 2-Azido-N,N-Dimethylethanamine and Tetramethylethylenediamine: From Drop Test to Gas-Phase Autoignition. Combust. Flame 2024, 268, 113590.

(29) Sun, C.; Tang, S. Hypergolic Ionic Mixtures with Task-Specific Ions: A New Strategy to Improve Performances of Ionic Liquids as Propellant Fuels. Combust. Flame 2021, 228, 107-113.

(30) Zhang, Z.; Zhao, Z.; Wang, B.; Zhang, J. Boron Based Hypergolic Ionic Liquids: A Review. Green Energy Environ. 2021, 6, 794-822.

(31) Sun, C.; Tang, S.; Zhang, X. Role of Cation Structures for Energetic Performance of Hypergolic Ionic Liquids. Energy Fuels 2017, 31, 10055-10059.

(32) Kopacz, W.; Okninski, A.; Kasztankiewicz, A.; Nowakowski, P.; Rarata, G.; Maksimowski, P. Hydrogen Peroxide – A Promising Oxidizer for Rocket Propulsion and Its Application in Solid Rocket Propellants. FirePhysChem 2022, 2, 56-66.

(33) Seo, M.; Bhosale, V. K.; Im, H.; Kwon, S. Performance Improvement of Triglyme-Based Fuels Using an Ionic Liquid With Hydrogen Peroxide. Combust. Flame. 2024, 270, 113719.

(34) Bhosale, V. K.; Jeong, J.; Choi, J.; Churchill, D. G.; Lee, Y.; Kwon, S. Additive-Promoted Hypergolic Ignition of Ionic Liquid with Hydrogen Peroxide. Combust. Flame. 2020, 214, 426-436.

(35) Bhosale, V. K.; Jeong, J.; Kwon, S. Ignition of Boron-Based Green Hypergolic Fuels with Hydrogen Peroxide. Fuel 2019, 255, 115729.

(36) Webber, W. T.; Hoffman, R. J. Mechanistic Model for Analysis of Pulse-Mode Engine Operation. J. Spacecr. Rockets 1975, 12, 113-117.

(37) Zhang, D.; Yu, D.; Zhang, P.; Yuan, Y.; Yue, L.; Zhang, T.; Fan, X. Hypergolic Ignition Modulated by Head-on Collision, Intermixing and Convective Cooling of Binary Droplets with Varying Sizes. Int. J. Heat Mass Transf. 2019, 139, 475-481.

(38) Biswas, S.; Antonov, I.; Fujioka, K.; Rizzo, G. L.; Chambreau, S. D.; Schneider, S.; Sun, R.; Kaiser, R. I. Unraveling the Initial Steps of the Ignition Chemistry of The Hypergolic Ionic Liquid 1-Ethyl-3-Methylimidazolium Cyanoborohydride ([EMIM][CBH]) with Nitric Acid (HNO3) Exploiting Chirped Pulse Triggered Droplet Merging. Phys. Chem. Chem. Phys. 2023, 25, 6602-6625.

(39) Biswas, S.; Fujioka, K.; Antonov, I.; Rizzo, G. L.; Chambreau, S. D.; Schneider, S.; Sun, R.; Kaiser, R. I. Hypergolic Ionic Liquids: To Be or Not to Be? Chem. Sci. 2024, 15, 1480-1487.

(40) Paul, D.; Biswas, S.; Yeom, H.; Na, K.; Pantoya, M. L.; Kaiser, R. I. Unraveling the Nanosheet Zeolite-Catalyzed Combustion of Aluminum Nanoparticles-Doped exo-Tetrahydrodicyclopentadiene (JP-10) Energetic Fuel. ACS Appl. Mater. Interfaces 2024, 16, 53938-53949.

(41) Rizzo, G. L.; Biswas, S.; Antonov, I.; Miller, K. K.; Pantoya, M. L.; Kaiser, R. I. Exotic Inverse Kinetic Isotopic Effect in the Thermal Decomposition of Levitated Aluminum Iodate Hexahydrate Particles. J. Phys. Chem. Lett. 2023, 14, 2722-2730.

(42) Rizzo, G. L.; Biswas, S.; Pantoya, M. L.; Kaiser, R. I. Unraveling the Ignition Chemistry of singly Levitated Aluminum Iodate Hexahydrate (AIH) Particles. Chem. Phys. Lett. 2024, 842, 141212.

(43) Rizzo, G. L.; Biswas, S.; Ka, D.; Zheng, X.; Kaiser, R. I. Untangling the Efficient Boron-Initialized Hydroxyl-Terminated Polybutadiene Combustion for High Energetic Solid Propulsion Systems. J. Phys. Chem. A 2025, 129, 288-300.

(44) Biswas, S.; Paul, D.; Mondal, K.; Kaiser, R. I. Simulating Atmospheric Freezing of Single Aqueous Droplets to Ice in a Cryogenically Cooled Ultrasonic Levitator. Proc. Natl. Acad. Sci. U.S. A. 2025, 122 (6), e2425543122.

(45) Fried, A.; Mathews, C. W. The Resonance Fluorescence of BO2 Excited by a Pulsed Dye-Laser. Chem. Phys. Lett. 1977, 52, 363-367.

(46) Johns, J. W. C. The Absorption Spectrum of BO2. Can. J. Phys. 1961, 39, 1738-1768.

(47) Baier, M. J.; McDonald, A. J.; Clements, K. A.; Goldenstein, C. S.; Son, S. F. High-Speed Multi-Spectral Imaging of the Hypergolic Ignition of Ammonia Borane. Proc. Combust. Inst. 2021, 38, 4433-4440.

(48) Kramida, A.; Ralchenko. Y.; Reader, J. and NIST ASD Team (2022). NIST Atomic Spectra Database (version 5.10), National Institute of Standards and Technology, Gaithersburg, MD. https://dx.doi.org/10.18434/T4W30F. (acccessed November 4, 2023).

(49) Dagaut, P.; Glarborg, P.; Alzueta, M. U. The Oxidation of Hydrogen Cyanide and Related Chemistry. Prog. Energy Combust. Sci. 2008, 34, 1-46.